Kết quả thử nghiệm ASCON HASH với data 2KB được ESP32 trả về máy tính thông qua UART 0:

ASCON-128 Benchmark on ESP32 (Dual Core)

CPU Frequency: 240 MHz

Testing with message size: 2000 bytes

Running on both cores...

=== Results from Core 1 ===

Encryption:

=== Results from Core 0 ===

Time: 578 microseconds

Throughput: 3.30 MB/s

Cycles/byte: 69.36

--------------------

Decryption:

Time: 581 microseconds

Throughput: 3.28 MB/s

Cycles/byte: 69.72

--------------------

Ascon-Hash:

Time: 1641 microseconds

Throughput: 1.16 MB/s

Cycles/byte: 196.92

--------------------

Core 1: Verification successful

Encryption:

Time: 578 microseconds

Throughput: 3.30 MB/s

Cycles/byte: 69.36

--------------------

Decryption:

Time: 581 microseconds

Throughput: 3.28 MB/s

Cycles/byte: 69.72

--------------------

Ascon-Hash:

Time: 1641 microseconds

Throughput: 1.16 MB/s

Cycles/byte: 196.92

--------------------

Core 0: Verification successful

Sensor data hash: 577e9a3b26444ee0584a9bcfaf79fede4f5246403ac4a83d1df3d455a633f2cbeda981b4da36b7d850a9daa9f57f5da0c1272eb85cbd04e34ffeb093deb7b6b8

=== Final System Performance ===

Core 0 - Enc: 3.30 MB/s, Dec: 3.28 MB/s, Hash: 1.16 MB/s, RAM used: 12656 bytes

Core 1 - Enc: 3.30 MB/s, Dec: 3.28 MB/s, Hash: 1.16 MB/s, RAM used: 6144 bytes

--------------------

Total Encryption Throughput: 6.60 MB/s

Total Decryption Throughput: 6.57 MB/s

Total Hash Throughput: 2.32 MB/s

Total Combined Throughput: 15.49 MB/s

RAM chiếm dụng.

- Core 0: ram_use = 12656  bytes

- Core 1: ram_use = 6144 bytes

Lý do Core 0 cao hơn: nó chạy và cấp phát đầu tiên → heap bị phân mảnh, cấp phát vùng lớn hơn, và có thể cả mutex + cấu trúc task cũng nằm trên core 0.
Tổng cộng hai core dùng ~18 KB RAM (heap internal).

Nhược điểm  của cách tối ưu .

| Tối ưu bạn đang dùng | Lợi ích | Nhược điểm/Nguy cơ |
| --- | --- | --- |
| Chạy song song 2 core | Tăng gấp đôi throughput | Tốn gấp đôi RAM (mỗi core cần buffer riêng), tăng độ phức tạp đồng bộ, dễ race condition khi in Serial |
| Tăng CPU lên 240 MHz | Tăng tốc độ xử lý | Tăng tiêu thụ điện, sinh nhiệt, có thể gây không ổn định nếu nguồn yếu |
| Tắt Wi-Fi + Bluetooth | Giải phóng RAM (~300 KB), tiết kiệm điện | Không dùng được Wi-Fi/BLE song song với benchmark |
| Cấp phát buffer lớn trên heap | Linh hoạt, dễ đổi kích thước | Gây phân mảnh heap, RAM không giải phóng kịp nếu lặp lại nhiều lần |
| portDISABLE_INTERRUPTS khi đo | Đo thời gian rất chính xác | Làm ngắt hệ thống tạm ngưng → không dùng được nếu có task thời gian thực cần chính xác |